Akhila Pesaru

U21457568

RF/Microwave Circuits 1

ADS Lab – 2

Microstrip Quadrature Coupler Design and Analysis

- The schematic circuit for ideal transmission line quadrature coupler is provided below.

The higher level schematic that references the ideal transmission line coupler schematic is shown in below figure.

The graphs for S21 and S31 with bandwidths greater than -4dB are provided below.

Frequency at greater than -4dB for S21 is plotted.

m2= 2.8GHz ; m1= 2.2GHz

Bandwidth = m2-m1= 2.8-2.2 = 0.6GHz

m3= 1.800GHz ; m4= 3.200GHz

Bandwidth = m4-m3= 3.2-1.8 = 1.4GHz

- Section 2

- (ii) The bandwidth for S21 plot when plotted for greater than or equal to -4dB is 0.6GHz

Bandwidth for S31 plot when plotted for greater than or equal to -4dB is 1.4GHz

(iii) The plot for S11 where the bandwidth over which the isolation is greater than or equal to 20dB is shown below.

Bandwidth = m6-m5 = 2.650-2.350= 0.3

The plot for S21 where the bandwidth over which the isolation is greater than or equal to 20dB is shown below.

Bandwidth = m2-m1= 3.750-1.250= 2.5GHz

The plot for S31 where the bandwidth over which the isolation is greater than or equal to 20dB is shown below.

Bandwidth = m4-m3= 5.450-4.450 = 1GHz

The plot for S41 where the bandwidth over which the isolation is greater than or equal to 20dB is shown below.

Bandwidth= m8-m7= 2.650-2.350= 0.3GHz

(v) Plot of phase for S21 and S31 is given below.

Phase difference for two frequencies of two plots is= -90-(-180) = 90deg

After disconnecting the termination port 4 and connecting a 50ohn resistor to that open port and simulate it.

Plot for the above circuit with 3 ports is as below.

The plot before removing the termination 4 is as below.

Here we can observe the difference as since we removed a port from the 4 port circuit, we only get the graph for those 3 ports. There is no graph for the 4th port like before.

Now change the resistor value to 0ohm and re-simulate the schematic.

We will get the plot for S11, S21, S31 as below.

Since there is no port 4 in this schematic there will be no graph for port 4 and there is no change with the centre frequency.

Isolation for port 1 remains same.

Section 3: Microstrip Quadrature Coupler

Generate the schematic of microstrip quadrature coupler.

Now generate a higher level schematic like before.

Simulate the above higher level schematic and plot the graphs for S55, S65, S75, S85.

Now plot the graphs of phase for the same S55, S65, S75, S85.